ASBESTOS REMOVAL PLAN

NOTE: This section was prepared by Christopher J. Eident, CIH, CSP, Mystic Air Quality Consultants, Inc., 1204 North Road, Groton, Connecticut 06340, (203) 449-8903. Licensed Asbestos Project Designer Number 00015 (State of Connecticut).

SUMMARY:   This section covers the removal of all asbestos containing roofing materials from the  gutter area at 237 high Street in Middletown, Connecticut. The roof samples were analyzed  by AmeriSci.

1.1      GENERAL

1.2      Scope - This specification covers the abatement of exposure to asbestos from the building materials that have previously been determined to contain asbestos.

1.2.1    Asbestos has been classified by the Federal Government as a carcinogenic (cancer producing) material.  To comply with governmental requirements and minimize employee exposure, controls are necessary wherever there is a potential for exposure to airborne fibers.

1.2.2    All work and work areas will be in conformance with the requirements of EPA AHERA regulations (40 CFR Part 763), NESHAPS regulations (40 CFR 61 Subpart M) OSHA regulations (1926.1101 (g) (11)), and Connecticut Department of Health Services standards for Asbestos Abatement, Section 19a-332a-1 to Section 19a-332a-13.

1.2.3    Deviations from this plan will require the written approval of the Owner.

1.3      DESCRIPTION OF WORK

1.3.1    Asbestos Removal:

The work involves the removal of the roof field tar from the gutter section (about 1,000 ft2) of the roof at the North College Building as asbestos waste from 237 High Street  in Middletown, Connecticut .

The materials are not friable and will not become friable by the nature of the removal.  After lightly misting the surfaces, the roofing will be cut into sections using hatchets or other sharp  tools (or other none dust generating methods),  placing the waste directly into 6 mil poly lined waste containers (or wrapped in two layers of 6 mil poly), labeled with asbestos waste labels, and lowered to the ground.  Any debris on the roof or adjacent ground will be HEPA vacuumed after each removal.

All removal work will be done by trained asbestos workers who have appropriate physical exams and experience.  Workers will use a minimum of half face negative pressure respirators with high efficiency filters and full body "Tyvek" coveralls.

A licensed Asbestos Project Monitor will be on-site every day until the asbestos removal of the day's work has been completed. The project monitor will document compliance with the plan.

1.3.1.1  The work specified herein will be the removal of asbestos containing materials by persons who are knowledgeable, qualified, and trained in the removal, treatment, handling, and disposal of asbestos-containing material, and the subsequent cleaning of the affected environment.  These persons must comply with Federal and State regulations which mandate work practices, and be capable of performing the work of this contract.

1.3.1.2  The contractor will supply all labor, materials, equipment, services, insurance and incidentals which are necessary or required to perform the work in accordance with the applicable governmental regulations and these specifications.

1.3.2   We will post signs in and around the Work Area to comply with OSHA standard 29 CFR 1910.1001 and 1926.1101.  Post one (1) copy of each of the following documents at the work site:

Title 29, Code of Federal Regulations, Part 1910.1001

and 1926.1101 OSHA Asbestos Standards

Title 40, Code of Federal Regulations, Part 6l,

Subparts A and B, National Emission Standard for

Hazardous Air Pollutants

In addition, the Abatement Contractor must notify all other building occupants and contractors at the site that an asbestos abatement is about to be performed and indicate what control measures are being taken in accordance with OSHA 1926.1101.

1.3.3  When and if  rental equipment is to be used in removal areas or to transport waste materials, a copy of the written notification provided to the rental company informing them of the nature of use of the rented equipment will be submitted to the Owner.

1.4.0     ASBESTOS REMOVAL

The contractor will spray asbestos materials with amended water, using airless spray equipment capable of providing a "mist" application to reduce the release of fibers.  The asbestos material will be sprayed with water mist containing a wetting agent to enhance penetration.  The wetting agent will be a commercial product produced specifically as an asbestos wetting agent.  A fine spray of the amended water will be applied to reduce fiber release preceding the removal of the asbestos material.

In order to maintain asbestos concentrations at a minimum, the wet asbestos will be removed in manageable sections. Materials will not be allowed to dry out.  Material drop will not exceed 8 feet.  For heights up to 15 feet provide inclined chutes or scaffolding to intercept drop.  For heights exceeding 15 feet provide enclosed dust-proof chutes.

The contractor will Place danger labels on containers in accordance with OSHA standard 29 CFR 1910.1001 (g) (2) if not already pre-printed on containers.

1.4    CLEAN-UP

The contractor must remove visible accumulations of asbestos material and debris.

If the Project Monitor finds visible accumulations of dust or bulk asbestos containing materials in the Work Area, the Contractor will repeat the cleaning until the work area is in compliance.

1.5     DISPOSAL OF ASBESTOS-CONTAINING MATERIALS

The asbestos materials will be packaged in impermeable dust tight containers (i.e. two heavy duty six (6) mil plastic bags or sealed fiber pack drums). The waste vehicle will be lined and sealed with 6 mil poly.:

All containers will be labeled in large legible letter:

DANGER

CONTAINS ASBESTOS FIBERS

AVOID CREATING DUST

CANCER AND LUNG DISEASE HAZARD

The labels will also have the DOT label and NESHAPS required generator identification and site label information.

The landfill accepting the wastes will be notified before shipping for scheduling to insure that adequate personnel and apparatus are available at the time of disposal; and the asbestos materials will be delivered in separate shipments. It will not be transported with any other materials. The contractor will not be paid until the signed waste manifest is received.

1.6 AIR MONITORING AND ANALYSIS

Air sampling will be conducted by the Contractor, as necessary, to assure that workers are using appropriate respiratory protection in accordance with OSHA Standard 1910.1001 and 1926.58.

Return to North College Updates